3x3 LED Matrix Control with Arduino

Objective:

Learn how to wire and program an Arduino to control a 3x3 LED matrix. This project will help you understand basic matrix display control, multiplexing, and using digital pins for row and column management.

Materials Needed:

- Arduino Uno (or any compatible Arduino board)

- 3x3 LED Matrix (common cathode or anode)

- Jumper Wires

- Resistors (220Ω to 330Ω, depending on the LED specifications)

- Breadboard (optional)

- Arduino IDE (for programming the Arduino)

Wiring Instructions:

1. Identify Matrix Pins:

A 3x3 LED matrix consists of 9 LEDs arranged in 3 rows and 3 columns. It has 6 pins — 3 for rows and 3 for columns.

2. Connect the Pins:

- Row Pins (R1, R2, R3): Connect these to Arduino digital pins (e.g., pins 2, 3, 4).

- Column Pins (C1, C2, C3): Connect these to Arduino digital pins (e.g., pins 5, 6, 7).

- Resistors: Place current-limiting resistors (220Ω or 330Ω) in series with the column pins to prevent excess current.

3. Arduino Pin Setup:

- Connect Row Pins (R1, R2, R3) to Arduino pins 2, 3, and 4.

- Connect Column Pins (C1, C2, C3) to Arduino pins 5, 6, and 7.

Note: Depending on your matrix type (common cathode or anode), the connection logic may change, but the pin mapping remains the same.

Arduino Code:

// Pin definitions for rows and columns

int rowPins[] = {2, 3, 4};   // Rows connected to pins 2, 3, 4

int colPins[] = {5, 6, 7};   // Columns connected to pins 5, 6, 7

// Define the pattern to display on the matrix (1 = LED on, 0 = LED off)

byte pattern[3][3] = {

{1, 0, 1},  // First row

{0, 1, 0},  // Second row

{1, 0, 1}   // Third row

};

void setup() {

// Set all row pins as output

for (int i = 0; i < 3; i++) {

pinMode(rowPins[i], OUTPUT);

}

// Set all column pins as output

for (int i = 0; i < 3; i++) {

pinMode(colPins[i], OUTPUT);

}

}

void loop() {

// Loop through each row and column to display the pattern

for (int row = 0; row < 3; row++) {

// Turn on the current row

digitalWrite(rowPins[row], HIGH);

for (int col = 0; col < 3; col++) {

// Turn on the LED in the current row and column if the pattern is 1

if (pattern[row][col] == 1) {

digitalWrite(colPins[col], HIGH);

} else {

digitalWrite(colPins[col], LOW);

}

}

delay(500); // Delay to make the LED visible

// Turn off the current row

digitalWrite(rowPins[row], LOW);

}

}

Activity Rubric

| Criteria | Exceptional (4) | Proficient (3) | Basic (2) | Inadequate (1) |
| --- | --- | --- | --- | --- |
| Circuit Integrity | Wiring is clean, organized, and logically color-coded. No loose connections. | Circuit is functional and mostly tidy. | Circuit works but wiring is messy or fragile. | Circuit does not function; major wiring errors. |
| Soldering/Assembly | All joints are secure; LEDs are perfectly aligned in a grid. | Secure joints; minor alignment issues. | Some "cold" joints or LEDs are significantly crooked. | Connections are failing; LEDs falling out. |
| Code Functionality | Code uses loops or arrays to display patterns (e.g., a "moving" dot). | Code successfully lights each LED individually. | Code is inefficient (long lists of digitalWrites). | Code fails to compile or run. |
| Documentation | Includes a clear schematic and a brief write-up of the logic used. | Includes a basic diagram of the setup. | Documentation is incomplete or lacks detail. | No documentation provided. |