Lab 2 Activity: The Dancing LED System

Objective: To design and implement a multi-LED sequential system ("Dancing LEDs") using an Arduino, demonstrating the relationship between hardware bus connections and software loop logic.

1. Materials Required

- 1 x Arduino Uno

- 8 x LEDs (preferably different colors)

- 8 x $220\Omega$ Resistors

- 1 x Breadboard

- 10-12 x Jumper Wires

2. Hardware Design (The "Bus" Setup)

In microprocessor systems, we often treat multiple pins as a single "port." Here, we will use Digital Pins 2 through 9 to create an 8-bit output bank.

Wiring Instructions:

- Common Ground: Connect the Arduino GND pin to the blue (negative) rail of the breadboard.

- LED Array: Place 8 LEDs in a row on the breadboard.

- Resistors: Connect a $220\Omega$ resistor from the Cathode (short leg) of each LED to the GND rail.

- Signal Pins: Connect Digital Pins 2, 3, 4, 5, 6, 7, 8, and 9 to the Anodes (long legs) of the LEDs in sequential order.

3. Software Design (The "Logic")

Instead of writing 16 lines of code to turn each LED on and off, we use For Loops—a fundamental concept in microprocessor programming for handling repetitive hardware tasks.

C++

void setup() {

// Initialize pins 2 through 9 as OUTPUT using a loop

for (int pin = 2; pin <= 9; pin++) {

pinMode(pin, OUTPUT);

}

}

void loop() {

// Pattern 1: The "Chase" (Left to Right)

for (int pin = 2; pin <= 9; pin++) {

digitalWrite(pin, HIGH); // Turn LED on

delay(100);              // Speed of the "dance"

digitalWrite(pin, LOW);  // Turn LED off

}

// Pattern 2: The "Return" (Right to Left)

for (int pin = 9; pin >= 2; pin--) {

digitalWrite(pin, HIGH);

delay(100);

digitalWrite(pin, LOW);

}

}

4. Performance Rubric (Scale 1–5)

| Criteria | 1 (Beginning) | 3 (Proficient) | 5 (Exemplary) |
| --- | --- | --- | --- |
| Circuit Integrity | LEDs wired without resistors or incorrect polarity. | All 8 LEDs wired correctly; stable connections. | Professional, neat wiring with consistent color-coding. |
| Logic Implementation | Used digitalWrite manually for every pin (inefficient). | Successfully implemented for loops for setup and loop. | Added custom patterns (e.g., blinking outside-in). |
| System Timing | Delay is too fast or too slow to see the "dance." | Timing is consistent and creates a smooth visual effect. | Used variables to allow "speed-up" or "slow-down" effects. |
| Troubleshooting | Requires help to identify a loose wire. | Can identify a faulty LED or loose jumper independently. | Can use the Serial Monitor to debug pin states. |

5. Critical Thinking / Lab Report Questions

- Hardware: Why is it important to use a resistor for each LED rather than one resistor for the entire group?

- Software: How would you modify the code to make the LEDs "dance" twice as fast?

- Systems Design: If you wanted to control 100 LEDs instead of 8, why would our current "one-pin-per-LED" method fail? (Hint: Think about available microprocessor pins).